实验九 RSA-AES加密与通讯

9.1 实验目的

RSA加密是一种非对称加密。可以在不直接传递密钥的情况下，完成解密。这能够确保信息的安全性，避免了直接传递密钥所造成的被破解的风险。

AES，高级加密标准，在密码学中又称Rijndael加密法，是美国联邦政府采用的一种区块加密标准。这个标准用来替代原先的DES，已经被多方分析且广为全世界所使用。

然而RSA加密虽然较为安全，但效率较低，只适合于加解密少量数据，AES加密虽然没有RSA那么安全，属于对称加密，但加解密速度很快，适合加解密大量数据。因此为了兼顾效率与安全性，我们选择RSA-AES混合加密。

注：下文与代码中的公私钥指的是RSA加密中的公私钥，秘钥特指AES加密中使用的秘钥。

整个加密传输过程分为以下几个步骤：

- 主机与小车启动通讯程序。

- 小车端读取或生成秘钥（公钥与私钥），并存储，将公钥发送至主机端。

- 主机端接收公钥，生成AES加密使用的私钥，使用公钥对私钥进行加密后，发送至小车端。

- 小车端接收加密后的秘钥，使用私钥进行解密，获取秘钥。

- 主机与小车端的数据使用AES秘钥进行加密。

9.2 实验要求

1、了解RSA-AES加密原理，并尝试对加密过程进行优化。

2、学习使用python的Crypto库对数据进行加解密。

3、合作验证加密是否成功，是否能够进行破解。

4、实验2人一组，课时：4学时

主机环境推荐：Linux虚拟机: Ubuntu20.04， ROS Noetic

小车系统：自带的raspberry pi os , ROS Kinetic（本程序对ROS kinetic 及其 python2进行了适配）

9.3 实验基础知识讲解

9.3.1 RSA加密介绍

1977年，三位数学家Rivest、Shamir 和 Adleman 设计了一种算法，可以实现非对称加密。这种算法用他们三个人的名字命名，叫做RSA算法。从那时直到现在，RSA算法一直是最广为使用的"非对称加密算法"。毫不夸张地说，只要有计算机网络的地方，就有RSA算法。

图3-1 非对称加密算法

非对称加密是通过两个密钥（公钥-私钥）来实现对数据的加密和解密的。公钥用于加密，私钥用于解密。

图3-2 RSA加密算法

RSA算法优点

不需要进行密钥传递，提高了安全性

可以进行数字签名认证

RSA算法缺点

加密解密效率不高，一般只适用于处理小量数据（如：密钥）

容易遭受小指数攻击

9.3.2 AES加密介绍

AES含义

AES，高级加密标准，在密码学中又称Rijndael加密法，是美国联邦政府采用的一种区块加密标准。这个标准用来替代原先的DES，已经被多方分析且广为全世界所使用。

严格地说，AES和Rijndael加密法并不完全一样（虽然在实际应用中二者可以互换），因为Rijndael加密法可以支持更大范围的区块和密钥长度：AES的区块长度固定为128 比特，密钥长度则可以是128，192或256比特；

而Rijndael使用的密钥和区块长度可以是32位的整数倍，以128位为下限，256比特为上限。包括AES-ECB，AES-CBC，AES-CTR，AES-OFB，AES-CFB。

AES加密原理

AES加密过程涉及到4种操作，分别是字节替代、行移位、列混淆和轮密钥加。解密过程分别为对应的逆操作。由于每一步操作都是可逆的，按照相反的顺序进行解密即可恢复明文。加解密中每轮的密钥分别由初始密钥扩展得到。算法中16个字节的明文、密文和轮密钥都以一个4x4的矩阵表示。

AES算法特点

运算速度快，安全性高，资源消耗少

9.3.3 pycryptodome库介绍

官方文档网址：https://pycryptodome.readthedocs.io/en/latest/，推荐看英文原版，如果想要快速上手可以直接看Example板块

以下举例一些常规用法：

- 功能包的引入

- # 导入AES加密模块

from Crypto.Cipher import AES

# 导入RSA加密模块

from Crypto.PublicKey import RSA

# 导入生成随机秘钥模块

from Crypto.Random import get_random_bytes

# 导入RSA加密方法模块

from Crypto.Cipher import  PKCS1_OAEP

- 使用AES进行加解密

- # 导入必要模块

- from Crypto.Cipher import AES

- from Crypto.Random import get_random_bytes

- # 原始数据,必须为二进制字节

- data = b'secret data'

- # 生成AES秘钥,长度为16个字节

- key = get_random_bytes(16)

- # 生成AES加密工具

cipher = AES.new(key, AES.MODE_EAX)

# 进行加密, ciphertext为加密后的信息， tag为加密特征码（解密时需要），nonce也是解密时需要

ciphertext, tag = cipher.encrypt_and_digest(data)

nonce = cipher.nonce

# 解密时需要秘钥，tag和nonce,三者缺一不可，需要在传输时进行封装

decrypt = AES.new(key, AES.MODE_EAX, nonce)

# 进行解密

data2 = decrypt.decrypt_and_verify(ciphertext, tag)

解密后数据恢复

- 生成RSA公私钥

- # 导入RSA库

- from Crypto.PublicKey import RSA

- # 生成RSA秘钥

- key = RSA.generate(1024)

- # 导出私钥

- private_key = key.export_key()

- # 导出公钥

- public_key = key.publickey().export_key()

- 生成的私钥和公钥如图所示

- 使用RSA-AES混合进行加解密

- 为了对任意大小的数据进行加解密，我们使用RSA-AES进行混合加解密

- 默认已经获取了前面生成的RSA的公私钥和AES的秘钥

# 导入库

from Crypto.PublicKey import RSA

from Crypto.Random import get_random_bytes

from Crypto.Cipher import AES, PKCS1_OAEP

# 原始数据

data = "I met aliens in UFO. Here is the map.".encode('utf-8')

# 对AES秘钥进行RSA加密

cipher_rsa = PKCS1_OAEP.new(RSA.import_key(public_key))

enc_key = cipher_rsa.encrypt(key)

# 对数据进行AES加密

cipher_aes = AES.new(key, AES.MODE_EAX)

ciphertext, tag = cipher_aes.encrypt_and_digest(data)

# 将加密后的秘钥（可以在建立连接时只传输一次），nonce，tag和加密后的数据一起传输出去

message = enc_key + cipher_aes.nonce + tag + ciphertext

解密：

假设已经获取了私钥

注意nonce和tage的长度均为16

# 导入库

from Crypto.PublicKey import RSA

from Crypto.Cipher import AES, PKCS1_OAEP

# 生成RSA解密工具

cipher_rsa = PKCS1_OAEP.new(RSA.import_key(private_key))

# 解密AES秘钥

key = cipher_rsa.decrypt(enc_key)

# 使用AES秘钥解密数据

cipher_aes = AES.new(key, AES.MODE_EAX, nonce)

data2 = cipher_aes.decrypt_and_verify(ciphertext, tag)

解密后

9.4 实验步骤

gitee仓库：https://gitee.com/IcathianRain/rsa--aes-encryption

9.4.1 主机端功能包

1、安装python PyCryptodome库

pip install pycryptodome

若安装较慢可使用清华源

pip install pycryptodome -i https://pypi.tuna.tsinghua.edu.cn/simple

- 编写功能包

- 文件结构

配置工作空间

mkdir -p catkin_ws/src

cd catkin_ws/src

catkin_init_workspace

创建主机端功能包

catkin_create_pkg secure_teleop_remote roscpp rospy std_msgs geometry_msgs message_generation

cd secure_teleop_remote

初始化功能包

mkdir scripts

mkdir launch

mkdir srv

- 在srv目录下新建sessionKey.srv和pubKey.srv

- sessionKey.srv文件内容

- uint8[] sessionKey

- ---

- bool flag0

- pubKey.srv文件内容

- uint8[] pubKey

- ---

- bool flag1

在launch目录下新建secure_teleop_remote.launch

在scripts目录下新建exchange_key.py和secure_teleop.py

修改CMakeLists.txt和package.xml与gitee仓库一致

第一次编译文件

返回到caktin_ws目录下

catkin_make

scripts/exchange_key.py 文件内容如下：

#!/usr/bin/env python3

# -*- coding: utf-8 -*-

import rospy  # 导入ROS模块

import os  # 导入os模块

from secure_teleop_remote.srv import * # 导入自定义的服务

from Crypto.Cipher import AES, PKCS1_OAEP

from Crypto.Random import get_random_bytes

from Crypto.PublicKey import RSA

# 全局变量声明

session_key = b''   # AES加密使用秘钥

enc_session_key = b''   # RSA加密后的秘钥

# 保存秘钥

def generate_and_store_session_key():

global session_key

# 生成秘钥

session_key = get_random_bytes(16)

# 保存秘钥至 ~/Key1/session_key.pem

with open('session_key.pem', 'wb') as e:

e.write(session_key)

# 接受小车传来的公钥

# 使用公钥对秘钥进行加密

def handle_receive_public_key(req: pubKeyRequest):

global enc_session_key

public_key = req.pubKey  # 获取公钥

print('成功接受公钥')

# 使用公钥对秘钥进行加密

recipient_key = RSA.import_key(public_key)

cipher_rsa = PKCS1_OAEP.new(recipient_key)

enc_session_key = cipher_rsa.encrypt(session_key)

return pubKeyResponse(True)

def send_enc_session_key():

# 将加密的密钥发送出去

rospy.wait_for_service('exchangeSessionKey')

try:

send = rospy.ServiceProxy('exchangeSessionKey', sessionKey)

msg = sessionKeyRequest()

msg.sessionKey = enc_session_key

resp = send(msg)

if(resp.flag0 == True):

print('成功发出秘钥')

except rospy.ServiceException as e:

print("Service call failed: %s" % e)

rospy.spin()

if __name__ == '__main__':

# 进入秘钥保存目录

home = os.path.expanduser("~")

try:

os.chdir(home + "/Key1")

except:

os.mkdir(home + "/Key1")

os.chdir(home + "/Key1")

# 初始化节点

rospy.init_node('remote_key')

generate_and_store_session_key()

# 接受公钥

s = rospy.Service('exchangePubKey', pubKey, handler=handle_receive_public_key)

print('等待公钥传输.....')

send_enc_session_key()

scripts/secure_teleop.py文件内容如下:

#!/usr/bin/env python3

# -*- coding: utf-8 -*-

# 开源说明

# Copyright (c) 2011, Willow Garage, Inc.

# All rights reserved.

#

# Redistribution and use in source and binary forms, with or without

# modification, are permitted provided that the following conditions are met:

#

#    * Redistributions of source code must retain the above copyright

#      notice, this list of conditions and the following disclaimer.

#    * Redistributions in binary form must reproduce the above copyright

#      notice, this list of conditions and the following disclaimer in the

#      documentation and/or other materials provided with the distribution.

#    * Neither the name of the Willow Garage, Inc. nor the names of its

#      contributors may be used to endorse or promote products derived from

#       this software without specific prior written permission.

#

# THIS SOFTWARE IS PROVIDED BY THE COPYRIGHT HOLDERS AND CONTRIBUTORS "AS IS"

# AND ANY EXPRESS OR IMPLIED WARRANTIES, INCLUDING, BUT NOT LIMITED TO, THE

# IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE

# ARE DISCLAIMED. IN NO EVENT SHALL THE COPYRIGHT OWNER OR CONTRIBUTORS BE

# LIABLE FOR ANY DIRECT, INDIRECT, INCIDENTAL, SPECIAL, EXEMPLARY, OR

# CONSEQUENTIAL DAMAGES (INCLUDING, BUT NOT LIMITED TO, PROCUREMENT OF

# SUBSTITUTE GOODS OR SERVICES; LOSS OF USE, DATA, OR PROFITS; OR BUSINESS

# INTERRUPTION) HOWEVER CAUSED AND ON ANY THEORY OF LIABILITY, WHETHER IN

# CONTRACT, STRICT LIABILITY, OR TORT (INCLUDING NEGLIGENCE OR OTHERWISE)

# ARISING IN ANY WAY OUT OF THE USE OF THIS SOFTWARE, EVEN IF ADVISED OF THE

# POSSIBILITY OF SUCH DAMAGE.

import rospy  # 导入 rospy 模块

import time

from std_msgs.msg import Int16MultiArray  # 导入 Int16MultiArray 消息类型, 存放密文

from geometry_msgs.msg import Twist  # 导入 Twist 消息类型， 存放机器人的速度

import sys, select, os  # 导入 sys, select, os 模块

if os.name == "nt":  # 如果是 Windows 系统

import msvcrt  # 导入 msvcrt 模块

else:  # 否则

import tty, termios  # 导入 tty, termios 模块

# 导入 AES 加密库

from Crypto.Cipher import AES

BURGER_MAX_LIN_VEL = 0.22

BURGER_MAX_ANG_VEL = 2.84

WAFFLE_MAX_LIN_VEL = 0.26

WAFFLE_MAX_ANG_VEL = 1.82

LIN_VEL_STEP_SIZE = 0.01

ANG_VEL_STEP_SIZE = 0.1

# 开始提示信息

start_msg = """

Start to connect and get the Public_key

-----------------------------------------

"""

# 收到秘钥后的提示信息

end_msg = """

Got the Public_key!

----------------------------------

"""

# 操控机器人的提示信息

msg = """

Control Your TurtleBot3!

---------------------------

Moving around:

w

a    s    d

x

w/x : increase/decrease linear velocity (Burger : ~ 0.22, Waffle and Waffle Pi : ~ 0.26)

a/d : increase/decrease angular velocity (Burger : ~ 2.84, Waffle and Waffle Pi : ~ 1.82)

space key, s : force stop

CTRL-C to quit

"""

e = """

Communications Failed

"""

def getKey():

if os.name == "nt":

if sys.version_info[0] >= 3:

return msvcrt.getch().decode()

else:

return msvcrt.getch()

tty.setraw(sys.stdin.fileno())

rlist, _, _ = select.select([sys.stdin], [], [], 0.1)

if rlist:

key = sys.stdin.read(1)

else:

key = ""

termios.tcsetattr(sys.stdin, termios.TCSADRAIN, settings)

return key

def vels(target_linear_vel, target_angular_vel):

return "currently:\tlinear vel %s\t angular vel %s " % (

target_linear_vel,

target_angular_vel,

)

def makeSimpleProfile(output, input, slop):

if input > output:

output = min(input, output + slop)

elif input < output:

output = max(input, output - slop)

else:

output = input

return output

def constrain(input, low, high):

if input < low:

input = low

elif input > high:

input = high

else:

input = input

return input

def checkLinearLimitVelocity(vel):

if turtlebot3_model == "burger":

vel = constrain(vel, -BURGER_MAX_LIN_VEL, BURGER_MAX_LIN_VEL)

elif turtlebot3_model == "waffle" or turtlebot3_model == "waffle_pi":

vel = constrain(vel, -WAFFLE_MAX_LIN_VEL, WAFFLE_MAX_LIN_VEL)

else:

vel = constrain(vel, -BURGER_MAX_LIN_VEL, BURGER_MAX_LIN_VEL)

return vel

def checkAngularLimitVelocity(vel):

if turtlebot3_model == "burger":

vel = constrain(vel, -BURGER_MAX_ANG_VEL, BURGER_MAX_ANG_VEL)

elif turtlebot3_model == "waffle" or turtlebot3_model == "waffle_pi":

vel = constrain(vel, -WAFFLE_MAX_ANG_VEL, WAFFLE_MAX_ANG_VEL)

else:

vel = constrain(vel, -BURGER_MAX_ANG_VEL, BURGER_MAX_ANG_VEL)

return vel

# 读取秘钥

def getSessionKey():

try:  # 尝试导入秘钥

home = os.path.expanduser("~")

# 打开公钥文件

with open(home + "/Key1/session_key.pem", "rb") as x:

session_key = x.read()  # 读取秘钥

except:

rospy.signal_shutdown("There is not a Session_key")  # 如果没有密钥，则退出

else:

return session_key  # 如果有密钥，则返回密钥

# 对msg进行加密

def Encrypt(msg, session_key):

cipher = AES.new(session_key, AES.MODE_EAX)

ciphertext, tag = cipher.encrypt_and_digest(msg)

# 密文为 nonce(长度16) + tag(长度16) + 加密后数据(长度未知)

ciphertext = cipher.nonce + tag + ciphertext

return (ciphertext)  # 返回加密内容

if __name__ == "__main__":

if os.name != "nt":

settings = termios.tcgetattr(sys.stdin)

rospy.init_node("secure_teleop")  # 初始化ROS节点

# 若success 不为True 则阻塞在这里

while rospy.get_param("success", False) != True:

time.sleep(1)

pass

# pub = rospy.Publisher('cmd_vel', Twist, queue_size=10)

print('秘钥交换成功')

# 关闭交换秘钥节点

os.system("rosnode kill /remote_key")

os.system("rosnode kill /robot_key")

# 创建 Publisher 发布密文

pub = rospy.Publisher("Cypered_cmd_vel", Int16MultiArray, queue_size=10)

# 设置模型

turtlebot3_model = rospy.get_param("model", "burger")

status = 0

target_linear_vel = 0.0

target_angular_vel = 0.0

control_linear_vel = 0.0

control_angular_vel = 0.0

try:

print(start_msg)

session_key = getSessionKey()

print(end_msg)

print(msg)

while 1:

key = getKey()

# 获取键盘输入

if key == "w":

target_linear_vel = checkLinearLimitVelocity(

target_linear_vel + LIN_VEL_STEP_SIZE

)

status = status + 1

print(vels(target_linear_vel, target_angular_vel))

elif key == "x":

target_linear_vel = checkLinearLimitVelocity(

target_linear_vel - LIN_VEL_STEP_SIZE

)

status = status + 1

print(vels(target_linear_vel, target_angular_vel))

elif key == "a":

target_angular_vel = checkAngularLimitVelocity(

target_angular_vel + ANG_VEL_STEP_SIZE

)

status = status + 1

print(vels(target_linear_vel, target_angular_vel))

elif key == "d":

target_angular_vel = checkAngularLimitVelocity(

target_angular_vel - ANG_VEL_STEP_SIZE

)

status = status + 1

print(vels(target_linear_vel, target_angular_vel))

elif key == " " or key == "s":

target_linear_vel = 0.0

control_linear_vel = 0.0

target_angular_vel = 0.0

control_angular_vel = 0.0

print(vels(target_linear_vel, target_angular_vel))

else:

if key == "\x03":

break

if status == 20:

print(msg)

status = 0

# 创建运动指令

twist = Twist()

# 设置线速度

control_linear_vel = makeSimpleProfile(

control_linear_vel, target_linear_vel, (LIN_VEL_STEP_SIZE / 2.0)

)

twist.linear.x = control_linear_vel

twist.linear.y = 0.0

twist.linear.z = 0.0

# 设置角速度

control_angular_vel = makeSimpleProfile(

control_angular_vel, target_angular_vel, (ANG_VEL_STEP_SIZE / 2.0)

)

twist.angular.x = 0.0

twist.angular.y = 0.0

twist.angular.z = control_angular_vel

# 创建运动信息 linear.x + " " + angular.z

str_msg = str(twist.linear.x) + " " + str(twist.angular.z)

print(str_msg)  # 打印运动信息

cipher_msg = Int16MultiArray()  # 创建密文载体

cipher_msg.data = list(Encrypt(str_msg.encode("ASCII"), session_key))  # 加密信息

pub.publish(cipher_msg)  # 发布密文

except:

print(e)

finally:

# 结束时，恢复静止状态

twist = Twist()

twist.linear.x = 0.0

twist.linear.y = 0.0

twist.linear.z = 0.0

twist.angular.x = 0.0

twist.angular.y = 0.0

twist.angular.z = 0.0

str_msg = str(twist.linear.x) + " " + str(twist.angular.z)  # 创建运动信息

cipher_msg = Int16MultiArray()  # 创建密文载体

cipher_msg.data = list(Encrypt(str_msg.encode('UTF-8'), session_key))  # 加密信息

print(cipher_msg.data)

pub.publish(cipher_msg)  # 发布密文

if os.name != "nt":  # 判断是否为windows系统

termios.tcsetattr(sys.stdin, termios.TCSADRAIN, settings)

launch/secure_teleop_key.launch文件内容如下:

<launch>

<arg name="model" default="$(env TURTLEBOT3_MODEL)" doc="model type [burger, waffle, waffle_pi]"/>

<param name="model" value="$(arg model)"/>

<param name="success" value="false" />

<node pkg="secure_teleop_remote" type="exchange_key.py" name="remote_key" output="screen">

</node>

<!-- turtlebot3_teleop_key already has its own built in velocity smoother -->

<node pkg="secure_teleop_remote" type="secure_teleop.py" name="secure_teleop_keyboard"  output="screen">

</node>

</launch>

9.4.2 小车端功能包

1、安装python PyCryptodome库

pip install pycryptodome

若安装较慢可使用清华源

pip install pycryptodome -i https://pypi.tuna.tsinghua.edu.cn/simple

- 编写功能包

- 若小车端编写代码困难，可在主机上编写代码后复制过去，故在主机的同一个工作空间内编写

- 文件结构

创建功能包

cd ~/catkin_ws/src

catkin_create_pkg secure_teleop_robot roscpp rospy std_msgs geometry_msgs message_generation

初始化功能包

mkdir scripts

mkdir launch

mkdir srv

在srv目录下新建sessionKey.srv和pubKey.srv

sessionKey.srv文件内容

uint8[] sessionKey

---

bool flag0

pubKey.srv文件内容

uint8[] pubKey

---

bool flag1

在launch目录下新建secure_teleop_robot.launch

在scripts目录下新建exchange_key.py和move.py

修改CMakeLists.txt和package.xml与gitee仓库一致

scripts/exchange_key.py 文件内容如下:

#!/usr/bin/env python

# -*- coding: utf-8 -*-

from cgitb import handler

import os

from Crypto.PublicKey import RSA

from secure_teleop_robot.srv import *

import rospy

from Crypto.Cipher import PKCS1_OAEP

private_key = b''

public_key = b''

session_key = b''

def generate_and_store_key():

global public_key

global private_key

# 生成或读取RSA公私钥

try:

f = open("public.pem", "rb")

except:

key = RSA.generate(1024)

private_key = key.export_key()

public_key = key.publickey().export_key()

with open("private.pem", "wb") as x:

x.write(private_key)

with open("public.pem", "wb") as x:

x.write(public_key)

f = open("public.pem", "rb")

else:

public_key = f.read()

f.close()

def send_pubKey(public_key):

rospy.wait_for_service('exchangePubKey')

print('正在发送公钥')

try:

send = rospy.ServiceProxy('exchangePubKey', pubKey)

msg = pubKeyRequest()

msg.pubKey = public_key

resp = send(msg)

return resp.flag1

except rospy.ServiceException as e:

print("Service call failed: %s" % e)

def handle_receive_sessionKey(req: sessionKeyRequest):

global private_key

print('已接收秘钥')

enc_session_key = req.sessionKey

# 获取撕咬

with open('private.pem', 'rb') as e:

private_key = RSA.import_key(e.read())

cipher_rsa = PKCS1_OAEP.new(private_key)

session_key = cipher_rsa.decrypt(enc_session_key)

# 保存秘钥

with open('session_key.pem', 'wb') as e:

e.write(session_key)

rospy.set_param("success", True)

return sessionKeyResponse(True)

if __name__ == '__main__':

# 进入公私钥与秘钥保存空间

home = os.path.expanduser("~")

try:

os.chdir(home + "/Key0")

except:

os.mkdir(home + "/Key0")

os.chdir(home + "/Key0")

# 初始化节点

rospy.init_node('robot_key')

# 生成公私钥并保存至 ~/Key0

generate_and_store_key()

# 发送公钥

send_pubKey(public_key)

# 使用私钥对秘钥进行解密

s = rospy.Service('exchangeSessionKey', sessionKey, handler=handle_receive_sessionKey)

print('等待秘钥传输')

rospy.spin()

scripts/move.py 文件内容如下:

#! /usr/bin/env python

# -*- coding: utf-8 -*-

import os

import rospy

from std_msgs.msg import Int16MultiArray

from geometry_msgs.msg import Twist

from Crypto.Cipher import AES

def get_session_key():

try:

home = os.path.expanduser("~")

with open(home + "/Key0/session_key.pem", "rb") as x:

session_key = x.read()

except:

rospy.signal_shutdown("There is not a Session_key")

else:

return session_key

def callback(cipher_text, arg):

pub = arg[0]

session_key = arg[1]

ciphermsg = bytearray(cipher_text.data)

print(ciphermsg)

# nonce 为密文0-15

nonce = ciphermsg[:16]

# tag 为密文16-31

tag = ciphermsg[16:32]

cipher_aes = AES.new(session_key, AES.MODE_EAX, nonce)

# print(nonce, tag)

# 密文为 32-最后

msg = cipher_aes.decrypt_and_verify(ciphermsg[32:], tag).decode('ASCII')

# print(msg)

x, z = msg.split(" ")

print("x:{}".format(float(x)))

print("z:{}".format(float(z)))

twist = Twist()

twist.linear.x = float(x)

twist.angular.z = float(z)

twist.linear.y = 0.0

twist.linear.z = 0.0

twist.angular.x = 0.0

twist.angular.y = 0.0

pub.publish(twist)

if __name__ == '__main__':

rospy.init_node('move')

# 等待结果

while rospy.get_param("success", False) != True:

pass

print('秘钥交换完成')

session_key = get_session_key()

pub = rospy.Publisher('cmd_vel', Twist, queue_size=10)

rospy.Subscriber("Cypered_cmd_vel", Int16MultiArray, callback,callback_args=(pub, session_key), queue_size=1)

rospy.spin()

launch/secure_teleop_robot.launch文件内容如下:

<launch>

<param name="success" value="false"/>

<node pkg="secure_teleop_robot" type="exchange_key.py" name="robot_key" output="screen">

</node>

<node pkg="secure_teleop_robot" type="move.py" name="move" output="screen">

</node>

</launch>

9.4.3运行程序

在主机端编译此功能包

cd ~/catkin_ws

catkin_make

source devel/setup.bash

运行主机launch文件

roslaunch secure_teleop_remote secure_teleop_key.launch

在小车端编译此功能包

cd ~/catkin_ws

catkin_make

source devel/setup.bash

运行主机launch文件

roslaunch secure_teleop_robot secure_teleop_robot.launch